# 室内定位公开数据集综述（特征 + 位置标签，按信号源分类）

本文件汇总了一批常用于科研的室内定位公开数据集，均为“特征向量 + 位置/轨迹标签”格式，并按主要信号源（WiFi、BLE、地磁、UWB、可见光、超声、RFID 等）进行分类。

| 类别 | 数据集名称 | 年份 | 信号 / 特征 | 标签 | 场景 / 规模（简要） | 下载链接 |
| --- | --- | --- | --- | --- | --- | --- |
| WiFi RSS | UJIIndoorLoc | 2014 | WiFi RSSI (520 APs) | BuildingID, Floor, X, Y | 3 buildings, 4–5 floors, ≈21k samples | https://archive.ics.uci.edu/dataset/310/ujiindoorloc |
| WiFi RSS | SODIndoorLoc | 2022 | WiFi RSSI (dense AP grid) | Building, Floor, (E,N), Scene, User, Phone | 3 buildings, 8000 m², 1.8k RPs, 24k+ samples | https://github.com/renwudao24/SODIndoorLoc |
| WiFi RSS | Long-Term WiFi Fingerprinting Dataset | 2018 | WiFi RSSI (monthly campaigns) | Local 2D coordinates | Bookshelf area, 15+ months repeated surveys | https://zenodo.org/records/1309317 |
| WiFi RSS | Wi-Fi Crowdsourced Fingerprinting (Tampere) | 2017 | WiFi RSSI (2.4/5 GHz) | Building-local 2D coordinates, device ID, time | 5-floor university building, 4648 fingerprints, 21 devices | https://zenodo.org/records/889798 |
| WiFi RSS | WLAN RSS Database for Fingerprinting | 2018 | WiFi RSSI | 2D coordinates (train/test paths) | 3-floor building, 2 radio maps (train/test) | https://zenodo.org/records/1161525 |
| WiFi RSS | Wi-Fi Positioning Dataset (multiusers & multidevices) | 2022 | WiFi RSSI (multi-device, multi-year) | 2D coordinates, building, scenario | 5 university buildings + COEX hall, 5 phones, 5-year span | https://www.kaggle.com/ashimran/wimest |
| WiFi RSS | TUJI1 Dataset (multi-device WiFi RSSI) | 2024 | WiFi RSSI (5 commercial devices) | 2D coordinates, device ID | Fine grid, dense RSSI sampling for device heterogeneity studies | https://zenodo.org/records/7641701 |
| WiFi RSS | WiFi RSSI Indoor Localization (Kaggle) | ≈2023 | WiFi RSSI | Reference point ID / coordinates | Single building, grid of RPs for fingerprinting | https://www.kaggle.com/datasets/brosnanyuen/wifi-rssi-indoor-localization |
| WiFi RSS | Wi-Fi Indoor Localization Dataset (WILD-v2) | 2021 | WiFi RSSI | Position / region labels (competition format) | Kaggle competition dataset for deployable WiFi localization | https://www.kaggle.com/c/wild-v2 |
| WiFi RSS | IPIN Smartphone-based Off-site Tracks (WiFi) | 2015–2023 | WiFi RSSI (+ sometimes inertial/others) | 2D/3D coordinates, floor, trajectories | Multiple competition venues, multi-floor smartphone data | http://indoorloc.uji.es/ipin2016track3/ |
| Hybrid (WiFi+Mag/IMU) | Geo-Magnetic Field and WLAN Dataset | 2016 | WiFi RSSI, geomagnetic, accel, gyro | 2D coordinates, path ID | Lab & corridor environment, wristband + smartphone | https://archive.ics.uci.edu/dataset/377/geo%2Bmagnetic%2Bfield%2Band%2Bwlan%2Bdataset%2Bfor%2Bindoor%2Blocalisation |
| Hybrid (WiFi+Mag/IMU) | Multisource & Multivariate WLAN+Mag Dataset (IPIN 2016) | 2016 | WiFi RSSI, geomagnetic, inertial | Symbolic and metric positions | Research labs, corridors, 185 m², multi-sensor fingerprints | https://wnet.isti.cnr.it/software/Ipin2016Dataset.html |
| Hybrid (WiFi+BLE+Mag) | Miskolc IIS Hybrid IPS | 2016 | WiFi, Bluetooth, magnetometer | Absolute (x,y,z) + symbolic locations | 3-floor IIS building, 1500+ records, hybrid positioning | https://archive.ics.uci.edu/ml/datasets/Miskolc%2BIIS%2BHybrid%2BIPS |
| Hybrid (WiFi+BLE+Mag) | Hybrid Indoor Positioning Dataset (WiFi+BLE+Mag) | 2016 | WiFi RSSI, BLE RSSI, magnetometer | Reference/area IDs | Indoor testbed, 1540 samples, 65-dim feature vectors | https://archive.ics.uci.edu/ml/datasets/Hybrid%2BIndoor%2BPositioning%2BDataset%2Bfrom%2BWiFi%2BRSSI%2C%2BBluetooth%2Band%2Bmagnetometer |
| Hybrid (WiFi+Mag/IMU) | MagWi (Magnetic + WiFi + IMU) | 2021 | Geomagnetic, WiFi RSSI, IMU, barometer | Local coordinates, building, user, orientation | Multi-building, multi-floor, 5 smartphones, long-term data | https://doi.org/10.21227/7g9v-6z48 |
| BLE RSS | BLE RSSI Dataset for Indoor Localization and Navigation | 2017 | BLE RSSI (beacons) | Zone / position identifiers | Building with BLE beacons, navigation & localization tasks | https://archive.ics.uci.edu/ml/datasets/BLE%2BRSSI%2BDataset%2Bfor%2BIndoor%2Blocalization%2Band%2BNavigation |
| BLE RSS | Indoor Bluetooth Low Energy Dataset (Localization, Tracking, Occupancy) | 2018 | BLE RSSI (fixed beacons, wearable tags) | Room/zone, trajectories, social interactions | Office-like environment, multi-person tracking | https://www.mdpi.com/1424-8220/18/12/4462 |
| BLE RSS | BLE RSS Measurements Dataset for Accurate Indoor Positioning | 2019 | BLE RSSI (raw & processed) | 2D coordinates, scenario IDs | Indoor testbed, >4700 BLE fingerprints | https://www.mdpi.com/2306-5729/4/1/12 |
| BLE RSS | Indoor Environment Dataset Based on RSSI (Bluetooth) | 2024 | BLE RSSI (multiple devices) | 2D coordinates, room IDs | Indoor IoT environment, multiple rooms, BLE anchors | https://data.mendeley.com/datasets/3v9mcsvcd2/1 |
| BLE RSS | Position-Annotated BLE RSSI Dataset | 2022 | BLE RSSI trajectories (high-precision GT) | 2D ground-truth trajectories (~4 cm median error) | Indoor environment with precise motion capture GT | https://www.kaggle.com/datasets/philotuxo/positionannotatedblerssidataset |
| BLE/WiFi/ZigBee RSS | RSSI Dataset for Indoor Localization Fingerprinting | 2020 | WiFi, BLE, ZigBee RSSI | Reference points / grid IDs | 3 buildings at Warsaw University of Technology | https://github.com/pspachos/RSSI-Dataset-for-Indoor-Localization-Fingerprinting |
| Magnetic | UJIIndoorLoc-Mag | 2015 | 3-axis magnetic field (time series) | Coordinates along paths / corridors | Lab corridors and rooms, magnetic fingerprinting | https://archive.ics.uci.edu/dataset/343/ujiindoorloc%2Bmag |
| Magnetic | MagPIE | 2017 | Geomagnetic field, IMU | Centimeter-level path trajectories | 3 campus buildings, handheld + robot mapping | https://doi.org/10.1109/IPIN.2017.8115961 |
| Magnetic | MagneFi (Multiuser Geomagnetic Dataset) | 2022 | Geomagnetic field, IMU | Building, floor, coordinates, user & device IDs | 3 buildings, multi-floor, multi-user magnetic fingerprints | https://www.techscience.com/cmc/v73n1/47753 |
| UWB | Indoor UWB Positioning and Position Tracking Data Set | 2023 | UWB ranges, CIR, timestamps | 2D/3D ground-truth trajectories | 4 indoor environments, static + dynamic scenarios | https://doi.org/10.5281/zenodo.8280736 |
| UWB (TDOA) | UTIL UWB Time-Difference-of-Arrival Dataset | 2022 | UWB TDOA, SNR, IMU, ToF laser, optical flow | Quadrotor 3D poses (motion capture) | Indoor flight experiments, 150 min total flight time | https://utiasdsl.github.io/util-uwb-dataset/ |
| UWB (single-anchor) | Single-Anchor Indoor Localization (DW1000) Dataset | 2018 | UWB CIR & derived features | Tag positions in 2 complex indoor environments | Single-anchor SALMA setup with directional antennas | https://www.tugraz.at/en/institutes/iti/salma/database |
| UWB (people flow) | Industrial People-Flow Dataset with UWB & MoCap | 2020 | UWB ranges, MoCap trajectories | Worker trajectories & activity labels | Industrial factory site, multi-worker movements | https://zenodo.org/records/7732563 |
| Visible Light | IS-based Visible Light Positioning Dataset | 2020 | Camera-captured ceiling LED patterns | Camera pose / position | Indoor LED lighting grid, camera-based VLP | https://www.kaggle.com/datasets/celsopereira1/visible-light-positioning-dataset |
| Visible Light | QUAPOS Visible Light Positioning Dataset | 2025 | Simulated PD RSS (quadrant photodiode) | 2D/3D receiver coordinates | 73 simulated VLP scenarios with different geometries | https://zenodo.org/records/14161525 |
| Ultrasound | Ultrasound Indoor Positioning Dataset | 2025 | Ultrasonic chirp signals / TOF | 2D receiver coordinates | Multiple rooms, 14–15 anchors, measured + simulated points | https://doi.org/10.5281/zenodo.14833224 |
| RFID | RFID Indoor Localization Using Statistical Features | 2021 | RFID RSSI & statistical features | Object / tag positions, room IDs | Full-scale smart home / indoor environment, millions of records | https://www.tandfonline.com/doi/abs/10.1080/01969722.2021.1932868 |